Application Note

Simple Open Real-Time Ethernet Gbit (SORTE_G)

Firmware API

Revision History

| Rev No | Revision History | Date |
| --- | --- | --- |
| 1.0 | initial version | 2/24/2023 |

Information in this document is subject to change without notice. Texas Instruments may have pending patent applications, trademarks, copyrights, or other intellectual property rights covering matter in this document.  The furnishing of this document is given for usage with Texas Instruments products only and does not give you any license to the intellectual property that might be contained within this document.  Texas Instruments makes no implied or expressed warranties in this document and is not responsible for the products based from this document

# TABLE OF CONTENTS

1	Introduction	4

2	ARM API	4

2.1	Controller	4

2.1.1	Controller Configuration	4

2.1.2	Source Code Example Configuration	5

2.1.3	Controller Process Data interface	5

2.2	Device	5

2.2.1	Device Configuration	5

2.2.2	Source Code Example Configuration	6

2.2.3	Device Process Data Interface	6

# TABLE OF FIGURES

No table of figures entries found.

# Introduction

This design implements a simple open real-time Ethernet Gbit (SORTE_G) with the programmable real-time unit and industrial communication subsystem (PRU-ICSS). SORTE protocol enables customer applications to exchange process data between the

Controller and devices in a 4-μs cycle time. The design contains open source PRU firmware to enable customers to differentiate their products. The SORTE protocol includes device discovery, parametrization, PHY and cable delay measurement, synchronization, and process data exchange.

# ARM API

This describes the ARM API.

## Controller

### Controller Configuration

Memory Location:

- PRU_ICSSG1_DRAM0_SLV_RAM

- Start Address: 0x030080000

- PARAM_DATA_OFFSET: 0x20

PARAM DATA description

| Name | Address Offset | Size | Description | Example |
| --- | --- | --- | --- | --- |
| Cycle_time | 0x00 | 4 bytes | SORTE_G Cycle time in nanoseconds | 10000 [ns] |
| Sync_time | 0x04 | 4 bytes |  | 0 |
| Delay_period | 0x08 | 4 bytes |  | 0 |
| Delay_burst | 0x0C | 1 byte |  | 16 |
| Delay_gap | 0x0D | 1 byte |  | 0 |
| Delay_forward_time | 0x0E | 2 bytes |  | 0 |
| Inter_packet_gap | 0x10 | 2 bytes |  | 96 [ns] |
| T_in | 0x12 | 2 bytes |  | 0 |
| T_out | 0x14 | 2 bytes |  | 0 |
| Topology | 0x15 | 1 byte |  | 0 |
| Diagnostics | 0x16 | 1 byte |  | 0 |
| Alarm | 0x17 | 1 byte |  | 0 |
| Crc_mode | 0x18 | 1 byte |  | 1 |

### Source Code Example Configuration

HW_WR_REG32(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Cycle_time, 10000);

HW_WR_REG32(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Sync_time, 0);

HW_WR_REG32(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Delay_period, 0);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Delay_burst, 16);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Delay_gap, 0);

HW_WR_REG16(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Delay_forward_time, 0);

HW_WR_REG16(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Inter_packet_gap, 96);

HW_WR_REG16(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + T_in, 0);

HW_WR_REG16(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + T_out, 0);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Topology, 0);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Diagnostics, 0);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Alarm, 0);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_DRAM0_SLV_RAM_REGS_BASE + PARAM_DATA_OFFSET + Crc_mode, 1);

### Controller Process Data interface

Memory Location

- PRU_ICSSG1_DRAM0_SLV_RAM

- Start Address: 0x030080000

- Process Data Offset: 0x700

## Device

### Device Configuration

Memory Location:

- PRU_ICSSG1_RAM_SLV_RAM

- Start Address: 0x030090000

Device config description

| Name | Address Offset | Size | Description | Example |
| --- | --- | --- | --- | --- |
| MDIO_PHY_CONFIG_OFFSET | 0x04 | 1 Byte | MDIO address PHY 1 | 3 |
| MDIO_PHY_CONFIG_OFFSET | 0x05 | 1 Byte | MDIO address PHY 2 | 15 |
| IOEX_INDATA_FRAME_BUFFER_LOCATION | 0x60 | 2 Byte | INDATA: device to controller data | 0x0000 |
| IOEX_OUTDATA_FRAME_BUFFER_LOCATION | 0x62 | 2 Byte | OUTDATA: controller to device | 0x0020 |
| MASTER_FRAME_BUFFER_LOCATION | 0x64 | 1 Byte | Frame buffer for the controller frame | 0x0040 |

### Source Code Example Configuration

HW_WR_REG16(icssgBaseAddr + CSL_ICSS_G_RAM_SLV_RAM_REGS_BASE + MDIO_PHY_CONFIG_OFFSET, (3<<0) | (15<<8));

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_RAM_SLV_RAM_REGS_BASE + IOEX_INDATA_FRAME_BUFFER_PTR, IOEX_INDATA_FRAME_BUFFER_LOCATION);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_RAM_SLV_RAM_REGS_BASE + IOEX_OUTDATA_FRAME_BUFFER_PTR, IOEX_OUTDATA_FRAME_BUFFER_LOCATION);

HW_WR_REG8(icssgBaseAddr + CSL_ICSS_G_RAM_SLV_RAM_REGS_BASE + MASTER_FRAME_BUFFER_PTR, MASTER_FRAME_BUFFER_LOCATION);

### Device Process Data Interface

Memory Location

- PRU_ICSSG1_DRAM1_SLV_RAM

- Start Address: 0x030082000

- Process Data Offset: 0x0040